Standard Operating Procedure Template

PE-06a  |  Process Excellence  |  Fit For Nuclear Toolkit

# How to use this template

Pages 2 and 3 are the blank SOP template: copy them into a new document for each procedure, complete every field, and delete the grey guidance text. Pages 4 and 5 are a completed worked example showing the standard expected.

An SOP sets out what to do, how to do it, and the checks to make for each work element. Write it at the workplace, with the people who do the job; one action per step; every key point carries its reason.

This is a controlled document format: register every SOP in the SOP Register (PE-06b), keep one live revision at the point of use, and treat printed copies as uncontrolled. When the process changes or a lesson is learned (PE-04), the SOP changes with it.

STANDARD OPERATING PROCEDURE

| Document number |  | Revision |  |
| --- | --- | --- | --- |
| Title |  |  |  |
| Area / process |  | Issue date |  |
| Author |  | Approved by |  |
| Next review date |  | Training required |  |
| Status | DRAFT / ISSUED / OBSOLETE |  |  |

# 1.  Purpose

Why this procedure exists: the output it produces and the risk it controls. One or two sentences.

# 2.  Scope

Which parts, processes, machines and people this SOP applies to, and anything explicitly excluded.

# 3.  Safety and PPE

Hazards specific to this task, required PPE, isolations or permits, and what to do before starting. If a risk assessment applies, reference it here.

# 4.  Tools, equipment and materials

Everything needed before starting: tools, gauges (with calibration status), consumables, documents and their revisions. Listing these here supports your 5S point-of-use storage (PE-05).

# 5.  Procedure

One action per step, in the order performed. Key points carry the how and the why: the tribal knowledge that makes the difference between done and done right. The check column records what is verified or written down at that step. Add photos or sketches wherever a picture beats a paragraph.

| Step | Action: what to do | Key point: how, and why it matters | Check / record |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |

# 6.  Quality checks and acceptance criteria

What is measured or inspected, with what, how often, against what limits, and where results are recorded. Align with your control plan (PE-09) and inspection records.

# 7.  If something is wrong: reaction plan

What the operator does on finding a nonconformance or abnormality: stop, contain, quarantine, who to tell, what to record. Link to your nonconformance process and Problem Register (PE-04a). Never leave 'what if it goes wrong' to judgement on the day.

# 8.  Revision history

| Rev | Date | Description of change and reason | Author / approver |
| --- | --- | --- | --- |

# 9.  Training: read and understood

Everyone who performs this task signs below after training against the current revision. Update the skills matrix accordingly.

| Name | Signature | Date |
| --- | --- | --- |

WORKED EXAMPLE  |  STANDARD OPERATING PROCEDURE

This example shows the expected standard of completion. It continues the toolkit's running example: the in-process bore check introduced as the countermeasure to reject P-001 (PE-04).

| Document number | SOP-014 | Revision | C |
| --- | --- | --- | --- |
| Title | In-process bore inspection, flange housings, CNC Cell 2 |  |  |
| Area / process | CNC Cell 2, finish boring | Issue date | 15 Jan 2026 |
| Author | S. Patel | Approved by | J. Whitfield, Quality Manager |
| Next review date | 15 Jan 2027 | Training required | Yes: all Cell 2 setters and operators |
| Status | ISSUED |  |  |

# 1.  Purpose

To detect boring bar insert wear before it produces an oversize bore, by measuring the bore in-process at defined tool-life points. Controls the recurrence of reject P-001.

# 2.  Scope

Applies to finish boring of all flange housing variants on CNC Cell 2, all shifts, all trained setters and operators. Does not cover first-off approval, which is SOP-009.

# 3.  Safety and PPE

Standard machining PPE (safety glasses, safety boots, no loose clothing). Machine must be in feed hold with spindle stopped before gauging. Risk assessment RA-CNC-02 applies.

# 4.  Tools, equipment and materials

Calibrated three-point bore gauge 60 to 70 mm (check the calibration label is in date before use); bore check record card QF-014 (current revision from the document station); component drawing at the revision stated on the route card; lint-free wipes.

# 5.  Procedure

| Step | Action: what to do | Key point: how, and why it matters | Check / record |
| --- | --- | --- | --- |
| 1 | Check the parts counter against the tool life card for the material in cut | Tool life is set in parts, per material grade; harder grades wear inserts faster. The check is due at 50% of tool life | Counter value on QF-014 |
| 2 | At the due point, complete the cut in progress, feed hold, stop the spindle | Never gauge with the spindle live | None |
| 3 | Wipe the bore clean with a lint-free wipe | Swarf or coolant under the gauge anvils reads undersize and hides wear | None |
| 4 | Measure the bore at top, middle and bottom of the last completed part | Three depths reveal taper from uneven insert wear, not just size drift | Three readings on QF-014 |
| 5 | Compare against the drawing limit minus the wear margin on the tool life card | The wear margin triggers an insert change before the limit is reached, not at it | Pass / change insert on QF-014 |
| 6 | If within margin: resume. If at or beyond margin: change the insert, re-zero, and re-check the next part | Changing early costs an insert; changing late costs the batch | Insert change logged on QF-014 |
| 7 | If any reading exceeds the drawing limit: stop, quarantine parts back to the last good check, and tell the team leader | This is the reaction plan in section 7: containment first, then root cause | Quarantine label and Problem Register entry |

# 6.  Quality checks and acceptance criteria

Bore diameter per component drawing, measured at three depths with the calibrated three-point gauge at 50% of tool life and at every insert change. All readings recorded on QF-014 and retained with the batch record.

# 7.  If something is wrong: reaction plan

Any reading over the drawing limit: stop the operation, quarantine all parts produced since the last passing check, label them with the quarantine tag, inform the team leader immediately, and raise the problem in the Problem Register (PE-04a). Do not restart until the cause is identified and the team leader authorises.

# 8.  Revision history

| Rev | Date | Description of change and reason | Author / approver |
| --- | --- | --- | --- |
| A | 12 Mar 2024 | First issue | T. Mason / J. Whitfield |
| B | 02 Oct 2025 | Gauge type updated following calibration review | S. Patel / J. Whitfield |
| C | 15 Jan 2026 | In-process check frequency changed to 50% of tool life; tool life now by parts count per material. Reason: root cause of bore reject P-001 (see PE-04a) | S. Patel / J. Whitfield |

# 9.  Training: read and understood

| Name | Signature | Date |
| --- | --- | --- |
| K. Adams | (signed) | 16 Jan 2026 |
| R. Bryce | (signed) | 16 Jan 2026 |
| D. Hughes | (signed) | 17 Jan 2026 |