HUMAN PERFORMANCE TOOLS REFERENCE CARD  |  NSS-05b  |  Use on every nuclear-significant task

Seven core tools from INPO 12-012. Not optional. Not bureaucracy. The barriers between a human error and a nuclear safety event.

| HP-1 PRE-JOB BRIEFING. 1. Identify task, scope, and sequence. 2. Confirm procedure revision and special instructions. 3. State nuclear safety significance and quality grade. 4. Identify error traps relevant to this task. 5. Confirm hold/witness points and notifications. 6. Discuss contingency actions. 7. Confirm stop-work criteria. 8. All attendees sign the briefing record. Use: Before every nuclear-significant task, especially Grade 1/2 and first-time tasks. |  | HP-2 SELF-CHECK (STAR). 1. STOP: pause, do not proceed on autopilot. 2. THINK: confirm the action and expected result. 3. ACT: perform deliberately with full attention. 4. REVIEW: check the result before moving on. Use: Before any safety-significant action, especially irreversible ones. At every hold point. After interruptions.. |  | HP-3 PEER CHECK. 1. Identify the specific action or result to check. 2. Checker evaluates independently, not by following along. 3. Checker confirms or challenges before action proceeds. 4. Any disagreement resolved before proceeding. 5. Both parties record their involvement. Use: Grade 1/2 activities, irreversible actions, after interruptions, when an error precursor is present. |  | HP-4 THREE-WAY COMMUNICATION. 1. Sender states information clearly. 2. Receiver repeats back in their own words. 3. Sender confirms or corrects the repeat-back. 4. If any uncertainty remains, repeat from the start. Use: Any safety-significant verbal instruction or information transfer: hold points, stop-work, procedure changes, shift handover. |
| --- | --- | --- | --- | --- | --- | --- |

| HP-5 PLACE KEEPING. 1. Mark each completed step on the procedure. 2. Never rely on memory alone. 3. Mark last completed step before any interruption. 4. Verify position from marking before resuming. 5. Never skip steps, even apparently unnecessary ones. Use: Any multi-step nuclear procedure. Essential where interruptions are likely.. |  | HP-6 CONCURRENT VERIFICATION. 1. Both individuals review the item independently. 2. Neither communicates their finding to the other first. 3. Both state findings simultaneously. 4. If findings agree: verified, record it. 5. If findings disagree: STOP, resolve before any action. Use: Critical measurements, safety settings, or system states where a single verification carries high consequence of error. |  | HP-7 STOP-WORK AUTHORITY. 1. Recognise: something does not seem right. 2. State: tell the relevant person clearly. 3. Stop: do not proceed past the point of concern. 4. Escalate: contact Nuclear Quality Lead or supervisor. 5. Wait: task does not restart until concern is resolved. 6. Record: log the stop-work event and resolution. Use: Any time. Any person. Any task. The threshold is a CONCERN, not certainty.. |  | COMMON ERROR TRAPS. 1. Time pressure. 2. Distraction or interruption. 3. Habit and complacency. 4. First time or infrequent task. 5. Overconfidence. 6. Unclear procedure. 7. Poor physical environment. 8. Simultaneous tasks. Spot a trap? Increase HP tool use. Do not proceed on autopilot.. |
| --- | --- | --- | --- | --- | --- | --- |

Refer to NSS-05a (Training Guide) for full detail on each tool  |  NSS-05c (Pre-Job Briefing Template)  |  NSS-05g (Fatigue and Fitness for Duty Policy)