TBT-08 Fire Safety and Emergency Procedures

| Ref | Version | Date | Duration | Audience |
| --- | --- | --- | --- | --- |
| TBT-08 | v1.0 | June 2025 | 15-20 minutes | All Employees |

Why This Matters

A fire can spread through a workshop in minutes. Knowing what to do, and doing it without hesitation, can save lives. In manufacturing environments, fuels, lubricants, flammable solvents, gas supplies, electrical systems, and wood waste all provide potential ignition sources and fuel. Every person on site has a role to play in fire prevention and in responding correctly when an alarm sounds.

Legal Background

The Regulatory Reform (Fire Safety) Order 2005 requires the Responsible Person to carry out a fire risk assessment, implement appropriate fire precautions, provide fire safety equipment and training, and ensure all employees know the emergency procedures. Employees must cooperate with fire safety requirements and not do anything that compromises fire safety.

Key Facts

- The three elements needed for fire are fuel, heat, and oxygen. Remove any one of these and the fire cannot start or continue. - Most workplace fires are preventable. Electrical faults, hot work without permits, and improper storage of flammable materials are leading causes. - Fire can double in size every minute. The priority is always evacuation, not firefighting. - Fire extinguishers are for tackling small, contained fires in the early stages only. Never put yourself at risk attempting to fight a fire. - Fire doors are critical. A closed fire door can hold back a fire for up to 30 minutes. Wedged-open fire doors remove this protection entirely.

| What You MUST Do | What You Must NOT Do |
| --- | --- |
| - Know the location of all fire exits, assembly points, and alarm call points in your area. - Know the type of fire extinguisher in your area and what classes of fire each is suitable for. - Follow the hot work permit procedure for all welding, grinding, and cutting operations. - Keep flammable materials stored correctly in designated, ventilated areas, away from ignition sources. - Keep fire exits and escape routes clear and unobstructed at all times. - Report any fire hazard, damaged fire equipment, or obstructed escape route immediately. - When the alarm sounds, leave immediately by the nearest safe exit. Do not stop to collect belongings. - Carry out fire evacuation drills seriously. They prepare you for real events. | - Do not wedge fire doors open. - Do not obstruct fire exits, fire extinguishers, or alarm call points with materials, equipment, or vehicles. - Do not use portable heaters in the workshop without authorisation and a fire risk assessment. - Do not store excessive quantities of flammable materials in the work area. - Do not re-enter a building after evacuation until an authorised person gives the all clear. - Do not allow rubbish, rags, or waste materials to accumulate near heat sources. - Do not attempt to fight a fire that has grown beyond its early stages or that is between you and the exit. |

Discussion Questions

1. Do you know where your nearest fire exit and assembly point are? Have you checked recently whether routes are clear? 2. Have you received training on how to use a fire extinguisher? What types do we have in our area, and what are they for? 3. What are the main fire risks in your work area? Are any of them currently not properly controlled? 4. How quickly do you think you could evacuate from your current work position? Are there any obstacles?

Key Takeaway

When the alarm sounds, go. Do not hesitate, do not collect belongings, and do not re-enter. Your life is never worth what you have left inside a building.

Attendance and Sign-Off Record

| Facilitator Name: | Date: | Location: |
| --- | --- | --- |

| # | Full Name (Print) | Job Title | Signature | Date |
| --- | --- | --- | --- | --- |

Facilitator Signature: ____________________________      Date: ________________

This record must be retained on file. A copy should be provided to HR / QHSE for central records.