TBT-09 Noise and Vibration

| Ref | Version | Date | Duration | Audience |
| --- | --- | --- | --- | --- |
| TBT-09 | v1.0 | June 2025 | 15-20 minutes | All Workshop / Production Staff |

Why This Matters

Noise-induced hearing loss and hand-arm vibration syndrome are two of the most common occupational health conditions in UK manufacturing. They are both permanent and both preventable. Unlike many other injuries, the damage builds up silently over months and years, meaning workers often do not realise the harm is occurring until it is too late. This talk covers the risks, the legal thresholds, and what controls must be in place.

Legal Background

The Control of Noise at Work Regulations 2005 require employers to take action when employees are exposed at or above 80 dB(A) (lower action value) and to provide hearing protection and exposure zones at or above 85 dB(A) (upper action value). The Control of Vibration at Work Regulations 2005 set equivalent thresholds for hand-arm and whole-body vibration. Employers must eliminate or reduce exposure to as low as reasonably practicable.

Key Facts

- Noise-induced hearing loss is permanent. Once the hair cells in the cochlea are damaged by noise, they cannot recover. - Tinnitus, a permanent ringing or buzzing in the ears, is one of the most common long-term effects of noise exposure. - Hand-arm vibration syndrome (HAVS) causes numbness, tingling, pain, and eventual loss of function in the hands and fingers. - Vibration white finger, a form of HAVS, causes reduced blood supply to the fingers, turning them white in cold conditions. - The daily vibration exposure action value is 2.5 m/s2 A(8). Many common tools such as grinders, impact drivers, and chipping hammers can reach this value in under an hour of continuous use.

| What You MUST Do | What You Must NOT Do |
| --- | --- |
| - Wear the correct hearing protection in designated Hearing Protection Zones and for noisy tasks. - Choose the correct hearing protection for the noise level in your area. Higher levels require more attenuation. - Use anti-vibration gloves and vibration-reduced tools where these are available and specified. - Take regular breaks from high-vibration tasks. Limit continuous use of vibrating tools. - Report any symptoms of hearing loss, tinnitus, or tingling and numbness in the hands to your supervisor. - Attend any noise monitoring or occupational health surveillance programmes arranged by the company. - Maintain vibrating tools correctly. Worn or unbalanced tools generate significantly higher vibration levels. | - Do not remove hearing protection in Hearing Protection Zones, even for short periods. Any unprotected exposure adds to the total daily dose. - Do not assume you can tolerate the noise because you have worked in it for years. Cumulative damage may already have occurred. - Do not use vibrating tools for longer continuous periods than the safe use time specified for that tool. - Do not ignore symptoms such as tingling, numbness, or a ringing in your ears after work. Report them early. - Do not allow hearing protection to become damaged or contaminated and continue to use it. |

Discussion Questions

1. Are there any areas in the workplace where you think the noise level could be reduced at source, rather than relying on hearing protection? 2. Have you ever experienced ringing in your ears after a shift? Did you report it? 3. Do you know the safe use times for the vibrating tools you regularly use? Are you working within them? 4. Does everyone in your area consistently wear hearing protection in the designated zones? If not, why not?

Key Takeaway

Noise and vibration damage builds silently. By the time you notice the symptoms, the damage is already done. Wear your protection consistently, take breaks, and report any symptoms early.

Attendance and Sign-Off Record

| Facilitator Name: | Date: | Location: |
| --- | --- | --- |

| # | Full Name (Print) | Job Title | Signature | Date |
| --- | --- | --- | --- | --- |

Facilitator Signature: ____________________________      Date: ________________

This record must be retained on file. A copy should be provided to HR / QHSE for central records.