TBT-COSHH-02 Welding Fume: A Group 1 Carcinogen

| Ref | Version | Date | Duration | Audience |
| --- | --- | --- | --- | --- |
| TBT-COSHH-02 | v1.0 | June 2025 | 15-20 minutes | Welders / Fabricators / Hot Work Operatives |

Why This Matters

In 2019, the International Agency for Research on Cancer (IARC) reclassified all welding fume as a Group 1 carcinogen. This is the same category as asbestos. It means there is now conclusive evidence that welding fume causes lung cancer. The law has changed as a result, and so must the way we work. This talk covers what has changed and what you must do differently.

Legal Background

The Control of Substances Hazardous to Health Regulations 2002 require employers to prevent or adequately control exposure to welding fume. Following the 2019 IARC reclassification, HSE updated its enforcement position: all welding fume (including mild steel) must be controlled using engineering controls, primarily Local Exhaust Ventilation (LEV). Relying on RPE alone without LEV is no longer acceptable practice indoors.

Key Facts

- All welding fume is now classified as a Group 1 carcinogen by IARC. This includes mild steel MIG and MMA welding, not just stainless or coated metals. - Welding fume contains iron oxide, manganese, silica, and in some processes, hexavalent chromium and nickel. Manganese causes permanent neurological damage (manganism) at sustained high exposures. - Metal fume fever from zinc oxide (galvanised steel welding) causes severe flu-like symptoms, typically on a Monday after a weekend away from exposure, and resolves over a day or two. It is not harmless. - Even where LEV is in use, RPE provides an additional layer of protection. Where LEV is not available or is inadequate, powered air-purifying respirators (PAPR) are required. - The LEV extraction nozzle must be positioned within approximately 300mm of the welding arc to be effective. Fume captured beyond this distance may bypass the hood.

| What You MUST Do | What You Must NOT Do |
| --- | --- |
| - Use LEV (fume extraction) for every indoor welding operation, no exceptions - Position the LEV extraction nozzle within 300mm of the welding arc - Switch LEV on before welding and leave it running after welding stops to clear residual fume - Wear RPE as specified in the COSHH assessment (minimum FFP3 for mild steel MIG/MMA where LEV is in use) - Use a Powered Air-Purifying Respirator (PAPR) where LEV is not available or where work is in a confined or restricted space - Report any cough, breathlessness, or chest symptoms to your supervisor and occupational health immediately - Attend annual lung function tests (spirometry) arranged as part of health surveillance - Check that your LEV unit is operating before each shift (indicator light, airflow gauge, or face velocity check) | - Do not weld indoors without LEV switched on and correctly positioned - Do not use compressed air to blow away welding fume from the work area - Do not assume stainless or coated metals are safe to weld without additional controls. They generate more hazardous fume than mild steel - Do not rely on general ventilation alone. It dilutes fume but does not remove it from the breathing zone - Do not remove your RPE until you have moved away from the welding area and fume has cleared - Do not ignore persistent cough or breathlessness. These can be early signs of occupational lung disease |

Discussion Questions

1. Did you know that welding fume has been reclassified as a Group 1 carcinogen? Has this changed how you think about LEV and RPE use? 2. Is the LEV in your welding area working correctly? When was it last tested? Do you know where the test records are kept? 3. What RPE are you issued for welding? Do you know the correct grade and whether it fits correctly? Have you had a face fit test? 4. If you were welding in a position where the LEV could not be positioned correctly, what would you do?

Key Takeaway

Welding fume is in the same cancer-risk category as asbestos. LEV on, hood within 300mm, RPE on. Every single weld, every single time. There is no acceptable level of exposure to a Group 1 carcinogen.

Attendance and Sign-Off Record

| Facilitator Name: | Date: | Location: |
| --- | --- | --- |

| # | Full Name (Print) | Job Title | Signature | Date |
| --- | --- | --- | --- | --- |

Facilitator Signature: ____________________________      Date: ________________

This record must be retained on file. A copy should be provided to HR / QHSE for central records.