TBT-COSHH-03 Solvents and Chemical Safety in the Workshop

| Ref | Version | Date | Duration | Audience |
| --- | --- | --- | --- | --- |
| TBT-COSHH-03 | v1.0 | June 2025 | 15-20 minutes | All Workshop / Production Staff |

Why This Matters

Solvents and chemical cleaning agents are used throughout manufacturing workshops for degreasing, surface preparation, and cleaning. Many workers regard them as routine substances because they have used them for years. That familiarity breeds complacency, and complacency with solvents can cause serious and permanent harm. This talk covers what you need to know about working safely with solvents and chemicals.

Legal Background

The Control of Substances Hazardous to Health Regulations 2002 apply to all solvents and chemical substances used at work. The employer must provide a COSHH assessment for each substance and you must follow the controls within it. Many solvents also have Workplace Exposure Limits (WELs) set in EH40 that must not be exceeded. Certain solvents are classified as carcinogens, reproductive toxins, or severe skin sensitisers.

Key Facts

- Many common workshop solvents such as isopropyl alcohol (IPA), trichloroethylene, and acetone pass through the skin and into the bloodstream, in addition to being inhaled as vapour. - Trichloroethylene and certain chlorinated solvents are classified as probable or confirmed human carcinogens. Their use is now heavily restricted in many workplaces under REACH regulations. - Chronic solvent exposure affects the central nervous system. Symptoms include memory difficulties, personality changes, balance problems, and cognitive decline — often before acute symptoms are noticed. - Solvents defat the skin, removing the natural protective oils. This is a significant cause of irritant contact dermatitis, particularly on the hands and forearms. - Flammable solvents such as IPA and acetone have low flash points and can be ignited by static electricity, grinding sparks, or even a light switch in enclosed spaces.

| What You MUST Do | What You Must NOT Do |
| --- | --- |
| - Read the COSHH assessment for every solvent you use before starting work - Work with solvents in a well-ventilated area or with LEV switched on - Keep solvent containers closed when not actively in use to reduce vapour buildup - Use the minimum quantity of solvent needed for the task - Store solvents in designated, ventilated, bunded COSHH cabinets away from heat sources and ignition risks - Wear nitrile or neoprene gloves as specified — check the breakthrough time for the specific solvent - Wear eye protection when there is any risk of solvent splash - Report any symptoms of headache, dizziness, or skin irritation after solvent use to your supervisor | - Do not use solvents to clean your skin. This removes protective oils and drives the chemical directly through the skin - Do not use solvents in enclosed, poorly ventilated areas without specific controls and RPE - Do not store large quantities of solvents in the workshop. Keep only the daily-use quantity on hand - Do not use solvents near naked flames, sparks, or hot surfaces if they are classified as flammable - Do not pour solvents down drains. Dispose of as chemical waste through your company's authorised waste stream - Do not assume a solvent is safe because it has a mild smell or because you have used it for years without obvious harm - Do not use a solvent not listed on the COSHH register without obtaining a COSHH assessment first |

Discussion Questions

1. Can you name all the solvents used in your area? Do you know where the COSHH assessments are for each one? 2. Are your solvents stored correctly in a COSHH cabinet? Are the quantities in the workshop limited to daily use quantities? 3. Have you ever felt dizzy, had a headache, or felt nauseous after working with solvents? Did you report it? 4. Do you know the difference between nitrile and neoprene gloves and when each is required for the solvents in your area?

Key Takeaway

Solvents are not routine. They are hazardous chemicals that enter your body through your skin, your lungs, and your eyes. Read the COSHH assessment. Use the controls. Wear the right PPE. Every time.

Attendance and Sign-Off Record

| Facilitator Name: | Date: | Location: |
| --- | --- | --- |

| # | Full Name (Print) | Job Title | Signature | Date |
| --- | --- | --- | --- | --- |

Facilitator Signature: ____________________________      Date: ________________

This record must be retained on file. A copy should be provided to HR / QHSE for central records.